from controller import Robot, GPS, Gyro, InertialUnit, Camera

import math, time, json

import paho.mqtt.client as mqtt

from ultralytics import YOLO

import numpy as np

import cv2

# -------- GLOBAL LEADER STATE --------

leader_x = None

leader_y = None

leader_z = None

# follower stays 1.2 m behind in WORLD frame (example)

OFFSET = [1, 0.5, 0.0]

def on_message(client, userdata, msg):

global leader_x, leader_y, leader_z

data = json.loads(msg.payload.decode())

if data["id"] == "leader":

leader_x = data["x"]

leader_y = data["y"]

leader_z = data["z"]

# -------- PID CONTROLLER --------

class PID:

def __init__(self):

self.px = self.py = 0

self.palt = self.pth = self.proll = 0

self.alt_i = 0

self.kp_xy = 1.5

self.kd_xy = 0.5

self.kp_rp = 0.5

self.kd_rp = 0.1

self.kp_z = 10

self.ki_z = 5

self.kd_z = 5

def clamp(self, a, lo, hi):

return max(lo, min(hi, a))

def compute(self, dt,

vx_des, vy_des, yaw_des, alt_des,

roll, pitch, yaw_rate, altitude,

vx, vy):

# Horizontal control

ex = vx_des - vx

ey = vy_des - vy

dx = (ex - self.px) / dt

dy = (ey - self.py) / dt

des_pitch = self.kp_xy * self.clamp(ex, -1, 1) + self.kd_xy * dx

des_roll  = -self.kp_xy * self.clamp(ey, -1, 1) - self.kd_xy * dy

self.px = ex

self.py = ey

# Altitude control

ez = alt_des - altitude

dz = (ez - self.palt) / dt

self.alt_i += ez * dt

thrust = 48 + self.kp_z * ez + self.kd_z * dz + self.ki_z * self.clamp(self.alt_i, -2, 2)

self.palt = ez

# Attitude control

ep = des_pitch - pitch

er = des_roll  - roll

dp = (ep - self.pth) / dt

dr = (er - self.proll) / dt

eyaw = yaw_des - yaw_rate

pitch_cmd = -(self.kp_rp * self.clamp(ep, -1, 1) + self.kd_rp * dp)

roll_cmd  =  (self.kp_rp * self.clamp(er, -1, 1) + self.kd_rp * dr)

yaw_cmd   = self.clamp(eyaw, -1, 1)

self.pth = ep

self.proll = er

m1 = thrust - roll_cmd + pitch_cmd + yaw_cmd

m2 = thrust - roll_cmd - pitch_cmd - yaw_cmd

m3 = thrust + roll_cmd - pitch_cmd + yaw_cmd

m4 = thrust + roll_cmd + pitch_cmd - yaw_cmd

return [m1, m2, m3, m4]

# -------- WEBOTS INIT --------

robot = Robot()

ts = int(robot.getBasicTimeStep())

# Enable camera

cam = robot.getDevice("camera")     # <-- must match camera name

cam.enable(ts)

# ---------------- LOAD YOLO MODEL ----------------

model = YOLO("yolov8n.pt")  # or your own .pt file

print("🔥 YOLO Model Loaded Successfully!")

imu = robot.getDevice("inertial_unit"); imu.enable(ts)

gps = robot.getDevice("gps"); gps.enable(ts)

gyro = robot.getDevice("gyro"); gyro.enable(ts)

m1 = robot.getDevice("m1_motor")

m2 = robot.getDevice("m2_motor")

m3 = robot.getDevice("m3_motor")

m4 = robot.getDevice("m4_motor")

for m in (m1, m2, m3, m4):

m.setPosition(float("inf"))

m.setVelocity(0)

client = mqtt.Client()

client.on_message = on_message

client.connect("test.mosquitto.org", 1883, 60)

client.subscribe("crazyflie/swarm")

client.loop_start()

pid = PID()

target_alt = 1.0

past_t = robot.getTime()

past_x = gps.getValues()[0]

past_y = gps.getValues()[1]

def clamp(v):

return max(0, min(120, v))

print("Follower Ready")

# -----------------------------------------------------------

#                      TAKEOFF SEQUENCE

# -----------------------------------------------------------

print("Taking off and waiting 5 seconds...")

takeoff_start = robot.getTime()

while robot.step(ts) != -1:

t = robot.getTime()

dt = t - past_t

if dt <= 0:

continue

# Stop takeoff after 5 seconds

if t - takeoff_start >= 5.0:

break

roll, pitch, yaw = imu.getRollPitchYaw()

yaw_rate = gyro.getValues()[2]

x, y, z = gps.getValues()

vx = (x - past_x) / dt

vy = (y - past_y) / dt

cosy = math.cos(yaw)

siny = math.sin(yaw)

vx_b = vx * cosy + vy * siny

vy_b = -vx * siny + vy * cosy

alt_des = 1.0

vx_des = 0

vy_des = 0

motors = pid.compute(dt,

vx_des, vy_des, 0, alt_des,

roll, pitch, yaw_rate, z,

vx_b, vy_b)

m1.setVelocity(-clamp(motors[0]))

m2.setVelocity( clamp(motors[1]))

m3.setVelocity(-clamp(motors[2]))

m4.setVelocity( clamp(motors[3]))

past_t = t

past_x = x

past_y = y

print("Takeoff complete. Starting Leader–Follower mode...\n")

# -----------------------------------------------------------

#                      MAIN FOLLOW LOOP

# -----------------------------------------------------------

while robot.step(ts) != -1:

# Grab Webots camera frame (BGRA format)

img = cam.getImage()

h = cam.getHeight()

w = cam.getWidth()

# Convert to numpy BGR

frame = np.frombuffer(img, dtype=np.uint8).reshape((h, w, 4))

frame = frame[:, :, :3]

#time.sleep(0.02)

# Run YOLO on the frame

results = model.predict(frame, conf=0.4, imgsz=640, verbose=False)

# Process detections

for box in results[0].boxes:

x1, y1, x2, y2 = box.xyxy[0].tolist()

cls_id = int(box.cls[0])

conf = float(box.conf[0])

name = model.names[cls_id]

#print(f"📦 Detected: {name} [{conf:.2f}] at {int(x1)}, {int(y1)}")

# Optional: Show annotated image

annotated = results[0].plot()

cv2.imshow("Crazyflie YOLO", annotated)

cv2.waitKey(1)

t = robot.getTime()

dt = t - past_t

if dt <= 0:

continue

roll, pitch, yaw = imu.getRollPitchYaw()

yaw_rate = gyro.getValues()[2]

x, y, z = gps.getValues()

vx = (x - past_x) / dt

vy = (y - past_y) / dt

cosy = math.cos(yaw)

siny = math.sin(yaw)

vx_b = vx * cosy + vy * siny

vy_b = -vx * siny + vy * cosy

# ------- FOLLOWER LOGIC (NO YAW ROTATION) -------

if leader_x is not None:

des_x = leader_x + OFFSET[0]

des_y = leader_y + OFFSET[1]

des_z = leader_z + OFFSET[2]

ex = des_x - x

ey = des_y - y

ez = des_z - z

vx_des = 1.0 * ex

vy_des = 1.0 * ey

dz_des = 1.0 * ez

else:

vx_des = vy_des = dz_des = 0  # hover until leader is known

target_alt += dz_des * dt

motors = pid.compute(dt,

vx_des, vy_des, 0, target_alt,

roll, pitch, yaw_rate, z,

vx_b, vy_b)

m1.setVelocity(-clamp(motors[0]))

m2.setVelocity( clamp(motors[1]))

m3.setVelocity(-clamp(motors[2]))

m4.setVelocity( clamp(motors[3]))

past_t = t

past_x = x

past_y = y

time.sleep(0.02)