from controller import Robot, GPS, Gyro, InertialUnit

import math, time, json

import paho.mqtt.client as mqtt

# ---------------- PID ----------------

class PID:

def __init__(self):

self.px = 0; self.py = 0

self.palt = 0; self.pth = 0; self.proll = 0

self.kp_xy = 2; self.kd_xy = 0.5

self.kp_rp = 0.5; self.kd_rp = 0.1

self.kp_z = 10; self.ki_z = 5; self.kd_z = 5

self.alt_i = 0

def clamp(self,a,b,c): return max(b,min(c,a))

def compute(self,dt, des_vx, des_vy, des_yaw_rate, des_alt,

roll,pitch,yaw_rate, altitude,

vx,vy):

ex = des_vx - vx

ey = des_vy - vy

dx = (ex - self.px) / dt

dy = (ey - self.py) / dt

des_pitch = self.kp_xy*self.clamp(ex,-1,1) + self.kd_xy*dx

des_roll  = -self.kp_xy*self.clamp(ey,-1,1) - self.kd_xy*dy

self.px = ex; self.py = ey

ez = des_alt - altitude

dz = (ez - self.palt) / dt

self.alt_i += ez*dt

thrust = 48 + self.kp_z*ez + self.kd_z*dz + self.ki_z*self.clamp(self.alt_i,-2,2)

self.palt = ez

ep = des_pitch - pitch

er = des_roll  - roll

dp = (ep - self.pth)/dt

dr = (er - self.proll)/dt

eyaw = des_yaw_rate - yaw_rate

pitch_cmd = -(self.kp_rp*self.clamp(ep,-1,1) + self.kd_rp*dp)

roll_cmd  =   self.kp_rp*self.clamp(er,-1,1) + self.kd_rp*dr

yaw_cmd   =   1.0 * self.clamp(eyaw,-1,1)

self.pth = ep; self.proll = er

m1 = thrust - roll_cmd + pitch_cmd + yaw_cmd

m2 = thrust - roll_cmd - pitch_cmd - yaw_cmd

m3 = thrust + roll_cmd - pitch_cmd + yaw_cmd

m4 = thrust + roll_cmd + pitch_cmd - yaw_cmd

return [m1,m2,m3,m4]

# ---------------- WEBOTS INIT ----------------

robot = Robot()

ts = int(robot.getBasicTimeStep())

imu = robot.getDevice("inertial_unit"); imu.enable(ts)

gps = robot.getDevice("gps"); gps.enable(ts)

gyro = robot.getDevice("gyro"); gyro.enable(ts)

m1=robot.getDevice("m1_motor");m2=robot.getDevice("m2_motor")

m3=robot.getDevice("m3_motor");m4=robot.getDevice("m4_motor")

for m in (m1,m2,m3,m4): m.setPosition(float("inf")); m.setVelocity(0)

client = mqtt.Client()

client.connect("test.mosquitto.org",1883,60)

pid = PID()

target_alt = 1.0

past_t = robot.getTime()

past_x = gps.getValues()[0]

past_y = gps.getValues()[1]

def clamp(a): return max(0,min(120,a))

print("Leader Ready")

# Waypoints

waypoints = [(0.0, 0.0, 2), (-60.2, -46.8, 2), (43.9, 31.3,2), (-54.3,  31.3, 2)]

waypoint_index = 0

# ---------------- MAIN LOOP ----------------

while robot.step(ts) != -1:

t = robot.getTime()

dt = t - past_t

if dt <= 0: continue

roll,pitch,yaw = imu.getRollPitchYaw()

yaw_rate = gyro.getValues()[2]

x,y,z = gps.getValues()

vx = (x-past_x)/dt

vy = (y-past_y)/dt

cosy = math.cos(yaw); siny = math.sin(yaw)

vx_b = vx*cosy + vy*siny

vy_b = -vx*siny + vy*cosy

# Waypoint following logic

if waypoint_index < len(waypoints):

wx, wy, wz = waypoints[waypoint_index]

dx = wx - x

dy = wy - y

dz = wz - z

dist = math.sqrt(dx**2 + dy**2 + dz**2)

if dist < 0.1:

waypoint_index += 1

continue

desired_yaw = math.atan2(dy, dx)

dyaw = desired_yaw - yaw

dyaw = math.atan2(math.sin(dyaw), math.cos(dyaw))  # normalize

dvx = 0.2 * math.cos(yaw) if abs(dyaw) < 0.1 else 0

dvy = 0.2 * math.sin(yaw) if abs(dyaw) < 0.1 else 0

target_alt = wz

else:

dvx = dvy = dyaw = 0

motors = pid.compute(dt,dvx,dvy,dyaw,target_alt,

roll,pitch,yaw_rate,z,

vx_b,vy_b)

m1.setVelocity(-clamp(motors[0]))

m2.setVelocity( clamp(motors[1]))

m3.setVelocity(-clamp(motors[2]))

m4.setVelocity( clamp(motors[3]))

msg = {"id":"leader","x":round(x,3),"y":round(y,3),"z":round(z,3)}

client.publish("crazyflie/swarm", json.dumps(msg))

past_t = t; past_x = x; past_y = y

time.sleep(0.02)